# Skyline Custom Reports

The Skyline Targeted Proteomics Environment provides informative visual displays of the raw mass spectrometer data you import into your Skyline documents.  It allows you to work with this data, to refine the peptides and transitions you are measuring and to tune integration boundaries.  A rich variety of calculated values and statistics may be exported from your Skyline document to a tabular comma separated value (CSV) file perfect for further analysis in tools like Excel and R.  The Skyline Results and Document Grids provide access to many of these values and allow you to edit custom annotations as you work with your data.

The first part of this tutorial will focus on Skyline custom reports.  You will learn how they can be designed, shared and used to export raw values and summary statistics from your Skyline documents.  Exported reports can be used in the following ways:

- Deeper statistical analysis in tools like Excel or R

- Broader statistical analysis across multiple experiments and/or laboratories

- Enforcing instrument quality control with summary statistics

- Providing an input format for other custom build software tools

- Providing an import format suited to relational data repositories

In developing Skyline we aim to create a vendor-neutral platform for targeted proteomics investigation.  Skyline can export similar custom reports for documents populated with the data from Agilent, AB Sciex, Bruker, Thermo-Scientific and Waters instruments.  The ability to create consistent reports across instrument platforms greatly facilitates cross-instrument comparisons and large multi-site studies.

Understanding how to create custom reports, tailored to your own experiments, is critical and described in detail in this Skyline custom reports tutorial.

The second part of this tutorial will focus on the Results and Document Grid views in Skyline, which provide real-time access to many of the data fields available in the custom reports.  You will learn to customize columns in the Results and Document Grids for immediate access to important values, as you inspect and refine your mass spectrometer output in Skyline.  You will also learn to work with custom annotations to provide a controlled vocabulary for further interpretation of and human interaction with your results, which can also be exported in custom reports.

# Getting Started

To start this tutorial, download the following ZIP file:

https://skyline.gs.washington.edu/tutorials/CustomReports.zip

Extract the files in it to a folder on your computer, like:

C:\Users\brendanx\Documents

This will create a new folder:

C:\Users\brendanx\Documents\CustomReports

Now open the Study7_example.sky file in this new folder by double-clicking, or by using Open on the File menu in a running instance of Skyline.

# Data Overview

The dataset in Study7_example.sky is a subset of a study that was performed by the Clinical Proteomics Technology Assessment for Cancer (CPTAC) consortium (T. Addona et al “Multi-site assessment of the precision and reproducibility of multiple reaction monitoring–based measurements of proteins in plasma”, Nature Biotechnology, 2009, 27, 633 - 641).  This particular study describes a calibration curve at different concentrations of analyte peptides and a constant spike in level of their corresponding isotope labeled internal standards.

The Skyline file displays data from a subset of CPTAC Study 7.2 data (in the paper referred to as “Study II”) acquired at one CPTAC site.  The Skyline file shows data for 10 different peptide sequences monitored by LC-MRM-MS with concentrations for the analyte proteins (see light peptides) ranging from concentrations at 500 fmol to 2.92 fmol (replicates J: 500 fmol; I: 275 fmol; H: 151 fmol; G: 83 fmol; F: 46 fmol; E: 25 fmol; and D: 8.55 fmol – lower concentration points although acquired are not included in this tutorial).  Heavy internal standard (IS) peptides are each spiked in at a concentration of 50 fmol.

To get a first impression what this dataset looks like perform the following steps:

- Click on the ‘HGFLPR’ peptide to highlight it.

- On the View menu, choose Peak Areas, and then click Replicate Comparisons.

A new window appears that shows peak area for each replicate.  The blue bars represent the heavy labeled version of this peptide (always spiked in at 50 fmol), and the red bars represent the light analyte at varying concentrations (each concentration point was acquired in technical quadruplicate).

Other tutorials describe how you can view and edit documents like this one.  For the purposes of this tutorial, however, the Study7_example.sky document has received an initial integration review, fully preparing if for reports to be exported and used in deeper statistical analysis.  Before continuing with the rest of the tutorial:

- Close the Peak Areas view.

# Creating a Simple Custom Report

To create your first Skyline custom report template, perform the following steps:

- On the File menu, choose Export, and click Report

An Export Report form will appear that will probably list at least the Skyline default report templates shown below:

Continue creating a new Skyline custom report template by performing the following steps:

- Click the Edit list button.

- In the Edit Reports form, click the Add button.

- In Edit Report form, enter ‘Overview’ in the Report Name field.

The form should now look like:

- Expand the field groups Peptides and Replicates by clicking on the ‘+’ button to see many more fields that can be exported.

- From the expanded Peptides group click the checkbox next to the Peptide Sequence field.

The form should now look like:

Note that the fields in the Edit Report form, shown above, have a distinct hierarchical structure.  At the highest level are the protein specific export fields which are shown at the bottom of the left box (Protein Name, Protein Description, Protein Sequence, Protein Note, and the expanded Replicates group below these that contains the general field Replicate Name).  Skyline custom reports also provide peptide specific export fields, contained in the expanded Peptides group at the top of the left box, such as Peptide Sequence and Average Measured Retention Time among others.  The peptide fields include a Peptide Results group, which is not expanded yet.  Also unexpanded is the Precursors group, which contains precursor specific export fields, a Precursors Results group, and finally a Transitions field group.  The Transitions group, in turn, contains the transition specific export fields and a corresponding Transition Results group, containing the most detailed results values, such as Retention Time, Fwhm, Start Time, End Time, Area and Background, from which many of the higher level results field values are derived.

In the Edit Report form continue to add fields to your first custom report template by performing the following steps:

- Expand the Precursors group (located directly below the expanded Peptides group at the very top) by clicking on the ’+’ icon.

- Expand the Precursor Results group field now visible in the expanded Precursor group.

- Click the checkbox next to the Isotope Label Type field from the expanded Precursors group.  (If necessary, scroll down or resize the form.)

- Repeat this procedure to add the following additional fields from the precursor Precursor Results group to the report template column list:

  - Best Retention Time - the RetentionTime value of the transition with the highest maximum intensity for the particular precursor

  - Total Area - the summed Area values of all individual transitions for the particular precursor (for more information about these field values, click the “?” button on the tool strip at the top of the Edit Report form)

- Check the Pivot Replicate Name check box in the lower left corner of the Edit Report form.

This should leave the Edit Report form looking like:

- Click the Preview button in the upper right corner of the Edit Report form.

The Preview form should appear looking like:

The Preview function is quite helpful when designing report templates.  At times it may even serve as a quick substitute for exporting a report to a file. The copy button () on the toolbar will copy all of the data including the column headers. You can also select all of the values in the Preview form by clicking the upper-left rectangle.  Pressing Ctrl-C, then copies the full report, excluding the column headers.  After copying the data using either method, it can be pasted into another tool, such as Excel.

The report preview displayed in part above, for replicates 7_2_D_01, and 7_2_D_02, shows precursor Best Retention Time and Total Area for all 10 peptides in both their light and heavy form across all LC-MRM-MS acquisitions/experiments in the columns to the right.  By scrolling to the right, you can see columns for all 28 LC-MRM-MS acquisitions.  Maximize the Preview form by clicking on the square in its upper right corner to see more columns.  In this example 7_2_D_01, and 7_2_D_02 are taken from samples at the same light analyte concentration.  Therefore, Total Area values are expected to be similar.  Each letter D through J indicates a different light analyte concentration.  Each concentration was acquired in technical quadruplicate, indicated by numbers 01 through 04. The heavy peptide concentrations were held constant at 50 fmol in all samples.

Close the Preview Report form by clicking on the ‘x’ in the upper right corner to return to the Edit Report form again.  To save this report template:

- Click the OK button.

You will see the new Overview report template appear in the Report template list.

- Click the OK button again to return to the Export Report form.

The new Overview report template is now added to the Skyline report template list.  The Export Report form should now look like:

From the Export Report form you can now export your new report by selecting ‘Overview’ in the Report list and clicking the Export button.  For now, click the Cancel button, dismissing all open forms, in order to start the process from the original Skyline document view.

# Exporting Report Data to a File

To export the report you created and previewed above to a CSV file, do the following:

On the File menu, choose Export, and click Report.

- Select ’Overview’ in the Report list of the Export Report.

- Click the Export button.

- Enter ‘Overview_Study7_example.csv’ in the File name field of the Export Report save form.

- Click the Save button.

If you switch to a Windows Explorer window, showing the CustomReports folder you created for this tutorial, you will see the Overview_Study7_example.csv file you just created.  Open it now with Excel and confirm that its contents are quite similar to the preview you displayed earlier.  Note that the file contains the column header names, which you cannot get by simply copying and pasting from the preview form.

# Sharing Report Templates

Report Templates in Skyline are very flexible, easy to manage and modify, and share with collaborators.

There are many reasons you might want to share a custom report template, whether you are participating in a large multi-laboratory study or working with a single collaborator, whether you are providing the proteomics community with your own custom tool that depends on particular fields from Skyline documents or providing supplementary information with a manuscript to allow others to repeat your protocols.  The Skyline support for instruments from multiple vendors means your shared report templates can be used to gather data from multiple sites with different instrumentation.

To share the Skyline report template you created in this tutorial, perform the following steps:

- On the File menu, choose Export, and click Report.

- Click the Share button in the Export Report form.

Skyline will present the Manage Views form, as shown below:

- Click on ‘Overview’ in the list to select it.

- Click the Share… button.

- Enter ‘Overview’ in the File name field of the Save As form.

- Click the Save button.

The report template is saved to the file Overview.skyr in the CustomReports folder created for this tutorial.   You could now share it with your collaborators, with your own analysis tool or as supplementary data for a manuscript.

# Managing Report Templates in Skyline

To gain an understanding of how others would use the custom report template you have now created and shared, you need to first delete it from your system.  Then you can add it back using the shared report file Overview.skyr that you created in the previous section, as if you had acquired this shared template from another source.

The Export Report form should still be open after the steps performed in the previous section.  To delete the ‘Overview’ report template from your Skyline settings, do the following:

- Click the Edit List button.

- Select the ‘Overview’ item in the list of the Manage Views form.

The Manage Views form should now look like:

To remove the ‘Overview’ item from the list, do the following:

- Click the Remove button.

- Click the OK button.

The Export Report form should now look like:

Just as when you first brought it up in this tutorial.

To demonstrate how your collaborators would import a new report template into Skyline follow the steps described below:

- Click the Edit List button to bring up the Manage Views form

- Click the Import button

- Select the ‘Overview.skyr’ file you created earlier in the Open form (which should still be showing the contents of the ‘CustomReports’ folder for this tutorial).

- Click the Open button.

- Click the OK button on the Manage Views form

These actions should add an ‘Overview’ item back to the Report list in the Export Report form.  To verify that this is indeed the report you saved, perform the following steps:

- Select the ‘Overview’ item in the Report list.

- Click the Preview button.

Skyline will present a Preview Report form containing the same values you saw earlier in this tutorial.

# Modifying Existing Report Templates

You can, of course, create much more data rich reports than the simple ‘Overview’ report you first created in this tutorial.  You can do this by creating another new report template, by editing your existing ‘Overview’ report, or, as shown below, by making a copy of the ‘Overview’ report and editing the copy.  To create a new, more complex report from a copy of the ‘Overview’ report template, perform the following steps:

- Click on Edit List button in the Export Report form.

- Select the ‘Overview’ item in the Report list.

- Click on the Copy button and choose Open View Editor on the menu that appears.

Skyline will present the Edit Report form with the fields from the ‘Overview’ report template already added, as shown below:

The report name for this new report template has not yet been assigned.  To assign it now:

- Enter ’Study 7’ in the View Name field.

Continue to add more fields as described below by clicking the checkbox next to each field to add each field to the column list for your report template, on the right side of the Edit Report form:

- Expand the field groups Peptides and Results by clicking the ‘+’ icons to their left.

- From the expanded Results group, add the following fields:

  - File Name – the name of the file from which the data was imported

  - Sample Name – the sample name, if the data was imported from a multi-sample WIFF file, or the file name again, if not

  - Replicate Name – the replicate name assigned to the data during import

- From the top level add Protein Name.

- From the expanded Peptide group, add the Average Measured Retention Time field – the average retention time over all replicates.

- Expand the Results group in the expanded Peptide group.

- From the expanded peptide Results group, add the following fields:

  - Peptide Retention Time – peptide RT for each replicate run

  - Ratio To Standard – area ratio of light to heavy

- Expand the Precursors group.

- From the expanded Precursors group, add the following fields:

  - Charge (displayed in the column list as PrecursorCharge)

  - Mz (displayed in the column list as PrecursorMz )

- Expand the Transitions group.

- From the expanded Transitions group, add the following fields:

  - Product Charge

  - Product Mz

  - Fragment Ion – the fragment ion name (e.g. y8, y10, b7, etc.)

- Expand the Results group in the Precursors group.

- From the expanded precursor Results group, add the following fields:

  - Max Fhwm - the maximum full width at half max (FWHM) of the transitions for the particular precursor

  - Min Start Time - elution time start at the baseline of a peak, which will be the same for all transitions (Q1/Q3) from the same precursor

  - Max End Time - elution time start at the baseline of a peak, which will be the same for all transitions (Q1/Q3) from the same precursor

- Expand the Results group in the Transitions group.

- From the expanded transition Results group, add the following fields:

  - Retention Time – retention time at the maximum intensity for a transition peak

  - Fwhm – full width at half max for the transition peak

  - Start Time – retention time at the starting transition peak integration boundary

  - End Time – retention time at the ending transition peak integration boundary

  - Area – area under the curve (AUC), minus background, for the transition peak

  - Height – maximum intensity for the transition peak

  - User Set Peak - indicates whether the peak is unchanged from when Skyline first extracted chromatograms (FALSE) or changed by the user (TRUE). Other values are possible for peaks that may have been change by importing a peak boundaries file, or reintegrating with a trained model.

- Also, uncheck the Pivot Replicate Name checkbox, at the bottom of the form.

The report template should now contain a lot more detail than the original ‘Overview’ report template, and the Edit Report form should look like:

To reorder some fields in this new report template, do the following:

- Select Protein Name in the column list on the right.

- Click the up arrow on the far right until the Protein Name field is at the top of the list.

You can achieve whatever ordering suits you best, in this way, using the up and down arrow buttons on the far right, and you can remove fields you may have added by mistake with the red ‘X’ button above the arrow buttons.  This tutorial, however, will continue without making any further changes to the column list.

To preview the values for the current document in the report template you have created, do the following:

- Click on the Preview button.

- When you have reviewed the data, close the preview form.

With data like this, where each peptide has both a light analyte and a matching heavy isotope labeled internal standard, it may be more convenient to work with the data for these paired precursors all in a single row to allow you to compare matching transition values more easily.

To make this change to the ‘Study 7’ report template, do the following:

- Check the Pivot Isotope Label check box.

- Click on the Preview button again to see how the report preview has changed.

All of the values for matching light and heavy transitions have been added to the same row.  New columns have been added to achieve this, and columns specific to an isotope label type have been given the prefixes ‘light’ and ‘heavy’.

The Preview form is shown below, scrolled to show the transition between ‘light’ and ‘heavy’ column names:

You might notice that the column ‘heavy IsotopeLabelType’ shows ‘heavy’ on every row, and that there is also a column ‘light IsotopeLabelType’ which shows ‘light’ on every column.  When you are pivoting on a column, it does not add much to include it in your column list.  Close the Preview form, and do the following to remove the ‘IsotopeLabelType’ field from this report:

- Select ‘IsotopeLabelType’ in the column list on the right.

- Click the red ‘X’ button on the far right edge of the column list.

You can click the Preview button again to verify that both the ‘light IsotopeLabelType’ and the ‘heavy IsotopeLabelType’ columns have been removed.  The next step in this tutorial, however, is to save your work by doing the following:

- Click the OK button in the Edit Report form.

- Click the OK button in the Edit Reports form.

This will return you to the Export Report form, which should now look like:

From here, you could now continue to export the new ‘Study 7’ report to a CSV file by selecting it and clicking the Export button, or click Share button to create file for sharing this new report.  For this tutorial, however, click the Cancel button to return to the ‘Study7_example’ document.

# Quality Control Summary Reports

So far you have created reports based on the values Skyline offers for each imported replicate of the peptides and transitions specified in a document.  Skyline also provides report fields with summary statistics across all imported replicates.  These summary fields are particularly suited to quality control by ensuring that replicate QC runs show good reproducibility and coefficients of variation (CV).  Skyline report templates can be designed with mean values and CV over all replicates for key parameters, such as peak area, FWHM and retention time.

This tutorial provides a summary report template that you will import into Skyline, but it also describes how to design such a summary report template in the Edit Report form.

To get started, first open a new document by doing the following:

- On the File menu, click Open.

- Navigate to the ‘CustomReports’ folder you created for this tutorial.  (may not be necessary)

- Select the ‘Study9S.sky’ file.

- Click the Open button.

This file contains data acquired by two CPTAC sites over 10 LC-MRM-MS runs, injecting 22 analyte peptides at a constant concentration of 50 fmol over all 5 runs.  Such a dataset could potentially be used to assess reproducibility of replicate injections.  In this part of the tutorial you will begin working with Live Reports, using the report to navigate around the document.  To do this you will use the Document Grid functionality instead of Export Report.

To view the data for key quality control metrics follow the steps below:

- On the View menu, choose Document Grid.

- At the top of the Document Grid form, click the Views button.  A dropdown list will appear.

- Choose Manage Views from the dropdown list.

- Click the import button in the Manage Views form

- Select the Summary_stats.skyr file from the ‘CustomReports’ folder in the Open form.

- Click the Open button.

A new view named ‘Summary Statistics’ should have been added to the View list, and the Manage Views form should now look like:

Click OK to dismiss the Manage Views form.

Then, on the Document Grid form, click the Views button and choose Summary Statistics

After you choose Summary Statistics, the document grid should look like the following:

This report shows that the data in Study9pilot.sky, taken from 5 replicate injections of the same sample, produce peptide CV values for FWHM mostly below 5% and Total Area in most cases under 10% as indicated for the corresponding columns with red boxes.  In addition, the column called Range Best Retention Time (red box) lists the RT drift in minutes for any specific peptide over the 5 replicate injections.  In this case there is only minimal RT drift (< 0.15 min, < 10 sec).

These summary reports can be used to flag quality control issues early, before they impact data acquired on important samples.  In this particular dataset, HPLC and MS seem to behave acceptably.  In one case, however, for the peptide INDISHTQSVSAK (from protein LEP), the CV value for Total Area is 59.2% and for FWHM is 23.5% (circled in red), significantly differing from the CV values of all the other peptides.

You can add a filter to this view so that only rows matching specific criteria are shown by doing the following:

- Choose Edit View from the Views dropdown.

- In order to find the CvTotalArea column in the tree view on the left, double click on that column in the list on the right.

The Customize View form should look like:

To define a new filter on this view:

- Click the Filter tab.  The Cv Total Area column will still be selected in the tree view on the left.

- Click the Add >> button to add that column to the list of filters.

- Set the Operation to Is Greater Than

- In the Value column, enter ‘.2’.

The Customize View form should look like:

Note that even though the Cv Total Area column is displayed as a percentage in the Document Grid, on the filter tab, the number “20%” is represented as “.2”.

- Click the OK button on the Customize View form to see the rows matching this filter.

To inspect this peptide more closely, do the following:

- Close the Document Grid by clicking the red X in the upper right corner.

- Select the peptide ‘INDISHTQSVSAK’ in the peptide tree view.

- On the View menu, choose Peak Areas, and click Replicate Comparison (F7).

The Peak Areas view, showing the total peak area and the contribution of each transition for all 5 replicates, help clarify the problem that was indicated in the CV values of the summary report preview.  For comparison, select several of the other peptides.  The Peak Areas view will show much better peak area reproducibility, as indicated by their much lower TotalArea CV values in the summary report preview, where the majority of the peptides had CV values < 10%.

Skyline summary reports can be used with quality control data to flag issues early, while Skyline itself provides powerful visual displays to help get to the root of the problem quickly.

# Results Grid View

In some cases, you may want to see some of the values presented in reports while you are editing the data, for immediate access to raw values displayed in the Skyline charts.  The Skyline Results Grid view provides real-time access to many of the data fields available in the custom reports.  In the final sections of this tutorial, you will learn to use the Results Grid to gain immediate access to important values and to annotate your data to capture important human insights, as you inspect and refine your mass spectrometer output in Skyline.

To get started using the Result Grid view, do the following:

- On the View menu, click Results Grid (Alt-F2).

Skyline should now look something like this:

Now rearrange the windows for better viewing, by doing the following:

- Click the Results Grid caption and drag it to the bottom edge of the main window to dock it.

- Click the Peak Areas caption and drag it to the right edge of the main window to dock it.

- Click and drag the splitters between the various window panes to adjust the space allocation.

- On the View menu, choose Auto-Zoom, and click Best Peak, if it is not already selected.

- Select the precursor element ‘467.2440+++’ below the currently selected peptide.

This should leave Skyline looking something like:

Skyline keeps its charts and the Results Grid in synch as you navigate the data.  If you click on one of the other rows in the Results Grid, Skyline changes the active chromatogram replicate tab.  It also changes which bar in the Peak Areas graph has the selection rectangle around it, as well as the selection in the Replicates list above the peptide tree view.  Now click on a different chromatogram chart tab, bar in the Peak Areas chart, or change the selection in the Replicates list.  The other views will update, including the selected row in the Results Grid.

The first column to the right of the Replicate column in the Results Grid is the Precursor Replicate Note.  This column allows you to associate a free text note with the PrecursorResults field group for the selected precursor and replicate.

- Type ‘Low signal’ in the first row with replicate name ‘Rep1’ for the currently selected peptide.

The Results Grid should now look like:

All of the other columns in the current Results Grid display are calculated by Skyline and cannot be edited, but you can change which columns get displayed and in what order.  Several of the columns are scrolled out of view in the image above.  To reduce the number of precursor results columns being displayed, do the following:

- From the Views dropdown at the top of the Results Grid, choose Edit View

- On the Customize View form, for each of the columns Min Start Time, Max End Time, and Library Dot Product, click on the column name in the list box, and then click on the X button to the right of the list box.

- Click the OK button.

You can now review the results for other precursors by doing the following:

- Click in the peptide tree view.

- Use the up and down arrow keys to change the selection to other precursor elements.

Finally expand a precursor and review its transitions.  You could apply the same technique to customize the columns being displayed for transitions.  Instead, continue now to the final section in this tutorial to learn how to add a custom annotation that you can control with the Results Grid, and later export in a Skyline report.

# Custom Annotations

Skyline supports three types of custom annotations:

- Free text (similar to the Note which is available by default on all types)

- Number

- True/False

- Value list

The last two types can be used to collect information in a controlled vocabulary, as you process your data, which can later be exported in a Skyline report and used as input for programmatic statistical analysis.

To define a simple new True/False annotation, perform the following steps:

- On the Settings menu, click Document Settings.

- Click the Edit List button on the Annotations tab of the Document Settings form.

- Click the Add button on the Define Annotations form.

- Type ‘Tailing’ in the Name field on the Define Annotation form.

- Select ‘True/False’ in the Type field.

- Check the Precursor Results checkbox in the Applies To list.

The Define Annotation from should now look like:

Now finish defining the annotation and add it to your document by doing the following:

- Click the OK button on the Define Annotation form.

- Click the OK button on the Define Annotations form.

- Check the checkbox for the new ‘Tailing’ annotation.

The Document Settings form should now look like:

- Click the OK button.

Select the second to last precursor element ‘564.7746++’ for the peptide ‘ESDTSYVSLK’.

The new Tailing column will not have been added to the custom view in the Results Grid, but you can add it:

- Choose Edit View from the Views dropdown at the top of the Results Grid.

- In the list view on the right, click on the third column down (“Precursor Peak Found Ratio”).  This will affect where in the list new columns get added.

- Click the checkbox next to Tailing at the bottom of the tree view on the left.

The Customize View form should now look like the following:

- Click the OK button.

You should see the new Tailing column added to the Results Grid between Precursor Replicate Note and Precursor Peak Found Ratio

The Skyline window should now look something like:

The peaks in this document do not have a lot of issues, but this one does have some peak tailing.  Check the checkbox in the new Tailing column for the ‘Rep1’ row.  Change selection to each of the other 5 rows.  Decide whether you think the tailing is worth noting and check the checkbox, if you think it is.  Change the selected item in the precursor tree and then return to the ‘564.7746++’ precursor to verify that your changes were recorded.

You can now export the new Tailing annotation in a Skyline custom report.  It will appear in the Precursor Results field group.

# Conclusion

This tutorial has introduced you to the flexible custom reports Skyline offers for accessing the rich set of values associated with your Skyline targeted proteomics documents.  These reports offer a smooth path from data analysis within Skyline to more complex statistical analysis with Excel or custom programs in R, Matlab, Java or C++.  You have learned about summary statistics available through these report templates, which can provide quick indicators of data quality.  You can share your custom report templates with collaborators or as supplementary material in your publications, helping others to repeat your analyses with new data sets even from different instrumentation, taking full advantage of the Skyline multi-vendor support.  Finally, you learned how to use the Skyline Results Grid to gain immediate access to many of the values available in Skyline reports and to work with custom annotations to add rich new information to your documents.  If you were not aware of these features before, they will certainly increase the scope of experiment you can achieve with Skyline.

# Further Study

Descriptions of the available columns are available in Skyline. If you hover the mouse over a column in the Document Grid or the Results Grid, a tooltip will appear with a description of the column. Also, in the Customize View form, there is a “?” button on the toolbar which brings up a page with a description of every column that is available.

It is possible to group rows together and perform statistics on them. For some examples of using this feature, see this page:

https://skyline.ms/wiki/home/software/Skyline/page.view?name=PivotEditor